Homework SET1

Mechanics of materials

Problem 1:

Two axial forces are applied to the bar shown below.

- Determine the internal force, PAB, in portion AB of the bar. Specify tension or compression.

- Determine the normal stress, σAB, in portion AB of the bar. Specify tension or compression.

Problem 2:

The U-shaped part is attached to the wall at the left. An overlapping glue joint, of length = 3 in. is used to joint the U-shaped part to a flat bar. A tensile axial load of P is applied to the flat bar. The thicknesses of the U-shaped part and the flat bar are both t, and the widths of both are w. The lengths of the U-shaped part and the flat bar are both L

t = 1 in. w = 4 in. = 3 in. L = 12 in. 􀁁

For the overlapping glue joint, the ultimate shear stress for the glue is 200 psi. If a factor of safety is 2.0 required, what is the maximum allowable load, P, which can be applied?

Problem 3

The piece of material, shown below, has undeformed dimensions of: Lx = 3 in., and Ly = 2 in., and Lz = 1 in. The Young’s modulus is E = 1*106 psi and Poisson’s ratio is ν=1/4 Two loads, Px and Py are applied, as shown below.

Lx = 3 in. psi 6110E=×

Ly = 2 in. ν= 1/4

Lz = 1 in.

Px = 300 lb. Py = 600 lb.

a) Determine the change length in the x–direction, δx, due to the applied forces

b) The change in length in the z–direction is defined as δz.

Problem 4

Determine the deformation of the steel rod shown under the given loads